Workflow Process of Machine Shop

1.Workflow Chart – Primary Machine Shop

### Primary Machine Shop – Workflow Steps

- Receive Wood from Raw Goods Area

  - Any wood issued from the wood store must be placed in the Raw Goods Area.

- Length Cutting – Cross Cut Machine

  - Cut planks to the required length as per the cutting list.

- Thickness Planning – DSP (Double Side Planner)

  - Process planks to achieve the required thickness.

- Moisture Check

  - Check moisture content of each plank and ensure it meets acceptable limits.

- Width Cutting – Rip Saw Machine

  - Cut planks to the required width as per the cutting list.

- Move to WIP Area

  - After rip saw cutting, planks must be moved to the WIP (Work-in-Progress) Area of the Primary Machine Shop.

- Send to Clamping / HF Press

  - If planks are combined to form wider panels, send them to Clamping or HF Press as applicable.

- Power Max Processing

  - All planks, which are wider in size, will move to the Power Max Machine for final thickness and sanding.

- Panel Saw Cutting

  - After Power Max processing, planks will move to the Panel Saw for final size cutting.

### Secondary Machine Shop – Workflow Steps

Process as per Cutting List

- After Panel Saw cutting, planks/components will move to the applicable machine as per the cutting list requirements.

  - Moulder – Used to create decorative or functional profiles on edges and surfaces.

  - Router – Used for shaping, detailing, grooving, or making complex design cuts.

  - Tenon Machine – Used to create tenons for strong wood joints.

  - Mortiser – Used to create mortises (slots/holes) for joint assembly.

  - Grooving Machine – Used to cut straight grooves or channels in the wood.

  - Spindle Moulder – Used for curved profiling and shaping operations.

  - Edge Bander  – Used to apply edge finish to panels.

  - Wide Belt Sander – Used to achieve smooth, uniform surfaces on wood panels by sanding with precision, ensuring accurate thickness and fine finishing.